HEATED PCB BEAM

Dual- and Single-Heater Experimental Setups

Technical description of the PCB geometry, heater configurations, temperature-sensor array, and Arduino-based control system

Document scope Setup 1 is the dual-heater beam with one 30 ohm heater at each end of the sensing beam. Setup 2 is the single-heater beam with one 15 ohm heater at one end. Both setups use the same general PCB beam construction and five-sensor temperature array.

# 1. System Overview

The experimental platform consists of a narrow, thermally isolated PCB beam instrumented with five temperature sensors. The beam is integrated into a larger holding PCB, but mechanical cutouts and the absence of broad copper pours in the surrounding frame reduce unwanted heat conduction away from the active structure.

Two heater configurations are used. Setup 1 includes heater regions at both ends of the beam and can be driven from either end or from both ends simultaneously. Setup 2 includes a single heater region at one end of the beam. Both assemblies connect to an Arduino-based controller through a custom interface PCB that measures the sensor temperatures, commands the heater PWM outputs, records telemetry, and enforces an over-temperature shutdown.

# 2. Configuration Summary

| Parameter | Setup 1 - Dual heater | Setup 2 - Single heater |
| --- | --- | --- |
| Heater locations | One heater region at each end of the beam | One heater region at one end of the beam |
| Resistors per heater | Two 15 ohm resistors in series | Two 7.5 ohm resistors in series |
| Resistance per heater | 30 ohm | 15 ohm |
| Number of independent heaters | 2 | 1 |
| Power per heater at 5 V | 0.83 W | 1.67 W |
| Maximum total heater power | 1.67 W with both heaters active | 1.67 W |
| Heating direction | From either end or from both ends | From the single heater toward the free end |

Table 1. Comparison of the two heated PCB beam configurations.

# 3. Common PCB Beam Construction

Both setups use approximately the same PCB beam geometry. The PCB is 0.8 mm thick and uses 1 oz copper on both sides. A narrow copper-covered beam approximately 5 mm wide extends between the heater region and the sensor-side end. In the single-heater assembly, the stated beam extension is approximately 37.5 mm from the edge of the 9.5 mm x 9.5 mm heater copper area.

Copper on the upper and lower surfaces forms the primary axial heat-conduction path. Multiple vias connect the two copper layers between the heater and temperature-sensor positions. These vias improve through-thickness thermal coupling and help the top and bottom copper layers behave as a combined thermal path.

The active beam and heater regions are surrounded by a larger holding PCB. Cutouts separate the beam from the frame, and no broad copper pour is used outside the beam and heater areas. Only thin sensor interconnection traces, approximately 6 mil wide, bridge the isolated regions. This limits parasitic conduction into the support PCB and concentrates heat flow along the intended beam path.

# 4. Common Temperature-Sensor Array

Five temperature sensors are mounted on the top side of the beam. The heater resistors are mounted on the opposite PCB side beneath their respective copper heating regions. The nominal sensor positions, measured from the primary heater reference used in the original geometry, are listed below.

| Sensor | Arduino input | Nominal position | Role in Setup 1 | Role in Setup 2 |
| --- | --- | --- | --- | --- |
| Beam 1 | A1 | 0 mm | Nearest Heater 1 | Nearest the single heater |
| Beam 2 | A2 | 8.89 mm | Near Heater 1 side | Near-field beam temperature |
| Beam 3 | A3 | 19.05 mm | Central beam region | Intermediate beam temperature |
| Beam 4 | A4 | 29.2 mm | Near Heater 2 side | Far-field beam temperature |
| Beam 5 | A5 | 39.3 mm | Nearest Heater 2 / opposite end | Temperature near the free end |

Table 2. Beam temperature channels and nominal measurement locations.

Geometry datum note The sensor coordinates are referenced to the heater location as originally specified, while the 37.5 mm beam dimension is referenced to the edge of the rectangular heater copper area. These datums should remain distinct in drawings and thermal models.

# 5. Setup 1 - Dual-Heater Beam

Setup 1 is the two-sided or dual-ended heater configuration. A 9.5 mm x 9.5 mm copper heating area is located at the first end of the beam. A second copper heating area of the same dimensions is located at the opposite, sensor-side end near Beam Sensor 5. Each copper area carries an independent resistive heater and is connected to the beam copper through the local copper pour and via field.

| Heater 1 region | Beam 1 | Beam 2 | Beam 3 | Beam 4 | Beam 5 / Heater 2 region |
| --- | --- | --- | --- | --- | --- |
| 30 ohm heater | 0 mm | 8.89 mm | 19.05 mm | 29.2 mm | 39.3 mm / 30 ohm heater |

Figure 1. Simplified axial arrangement of Setup 1. The exact mechanical reference of the second heater should follow the PCB drawing.

## 5.1 Heater Electrical Characteristics

Each heater consists of two 15 ohm resistors connected in series. The nominal resistance of each heater section is:

Rheater = 15 + 15 = 30 ohm

With a 5 V supply, one active heater draws approximately:

Iheater = V / R = 5 / 30 = 0.167 A

The nominal power generated by one heater at 100% PWM duty is:

Pheater = V^2 / R = 5^2 / 30 = 0.83 W

Each resistor in an active heater dissipates approximately 0.42 W. When both independent heaters operate at full power, the total nominal current is approximately 0.333 A and the total heater power is approximately 1.67 W.

## 5.2 Operating Modes

• Heater 1 only: heat propagates from the Beam 1 end toward Beam 5.

• Heater 2 only: the primary heat-flow direction is reversed and heat propagates from the Beam 5 end toward Beam 1.

• Both heaters at equal duty: heat is introduced from both ends, producing a more symmetric temperature field and a central thermal-balance region.

• Both heaters at unequal duty: an intentionally asymmetric temperature profile can be generated and controlled.

For independent PWM duty fractions D1 and D2, the approximate average heater powers are 0.83D1 W and 0.83D2 W. The maximum combined power is therefore the same nominal 1.67 W as the single-heater configuration, but it is distributed between two ends of the beam.

# 6. Setup 2 - Single-Heater Beam

Setup 2 is the original single-sided heater configuration. The heat source is located on one 9.5 mm x 9.5 mm copper area at the Beam 1 end. The two heater resistors are installed on the bottom side of the PCB, while the five temperature sensors are installed on the top side. Heat spreads through the via-stitched copper region and then conducts along the narrow beam toward Beam Sensor 5 and the free end.

| Heater region | Beam 1 | Beam 2 | Beam 3 | Beam 4 | Beam 5 / free end |
| --- | --- | --- | --- | --- | --- |
| 15 ohm heater | 0 mm | 8.89 mm | 19.05 mm | 29.2 mm | 39.3 mm |

Figure 2. Simplified axial arrangement of Setup 2.

## 6.1 Heater Electrical Characteristics

The single heater consists of two 7.5 ohm resistors connected in series, giving a nominal total resistance of:

Rheater = 7.5 + 7.5 = 15 ohm

With a 5 V supply, the nominal full-power current is:

I = V / R = 5 / 15 = 0.333 A

The nominal total heater power at 100% PWM duty is:

P = V^2 / R = 5^2 / 15 = 1.67 W

For two equal series resistors, each resistor dissipates approximately 0.83 W at full power. Actual power in either setup may differ because of supply-voltage tolerance, wiring and transistor losses, resistor tolerance, and resistance variation with temperature.

# 7. Temperature Measurement Electronics

The Arduino firmware is configured for one ambient-temperature sensor and five beam-temperature sensors. The ambient sensor is connected to A0, and Beam Sensors 1 through 5 are connected to A1 through A5. The ambient sensor is located on or near the custom controller PCB and therefore represents the local controller-area temperature rather than necessarily the exact air temperature adjacent to the complete beam.

The firmware is configured for MCP9701A analog temperature sensors and uses a 3.3 V external analog reference. It requests 14-bit ADC conversion and averages 16 ADC readings for each reported temperature value. The implemented nominal transfer relationship is:

Vout = 0.400 V + (0.0195 V/deg C) x T

The corresponding temperature conversion is:

T = (Vout - 0.400) / 0.0195

# 8. Heater, Fan, and Controller I/O

| Function | Pin | Firmware behavior | Use in Setup 1 | Use in Setup 2 |
| --- | --- | --- | --- | --- |
| Heater 1 PWM | D9 | 10 kHz, 0-100% duty | Drives first 30 ohm heater | Drives the 15 ohm heater |
| Heater 2 PWM | D10 | 10 kHz, 0-100% duty | Drives second 30 ohm heater | Unused or available |
| Fan PWM | D11 | 10 kHz, 0-100% duty | Optional forced cooling | Optional forced cooling |
| Fan tachometer | D2 | Falling-edge interrupt; 2 pulses/revolution | Fan-speed feedback | Fan-speed feedback |

Table 3. Controller outputs and tachometer input for both setups.

## 8.1 PWM Power Interpretation

For an ideal fixed-voltage resistive heater, the time-averaged power is approximately proportional to PWM duty fraction:

Pavg = D x V^2 / R

For Setup 1, each 30 ohm heater produces approximately 0.83D W. For Setup 2, the 15 ohm heater produces approximately 1.67D W. Here, D is expressed as a fraction from 0 to 1. The calculation assumes that the heater receives the full 5 V during every PWM on-period.

# 9. Data Acquisition and Safety

The controller evaluates the sensors continuously and generates telemetry at a nominal interval of approximately 250 ms, corresponding to about four records per second. The logged signals include elapsed time, ambient temperature, Beam 1 through Beam 5 temperatures, fan speed, Heater 1 PWM, Heater 2 PWM, and fan PWM. The two heater command fields allow Setup 1 to be analyzed under independent or simultaneous heating.

The controller compares the five beam temperatures and identifies the maximum measured value. If any beam sensor reaches 125 deg C, the firmware enters a fault state, forces both heater outputs to 0%, and commands the fan to 100%. Heater restart remains inhibited until the fault is explicitly cleared through the control interface.

Over-temperature response Threshold: 125 deg C ¦ Heater 1: OFF ¦ Heater 2: OFF ¦ Fan: 100% ¦ Restart: inhibited until fault clear

# 10. Thermal Measurement Principle

In Setup 1, energizing only Heater 1 or only Heater 2 permits heat propagation to be measured in opposite directions using the same sensor array. Driving both heaters permits symmetric and asymmetric dual-source temperature fields to be studied. The central sensors can be used to observe the interaction between the two thermal fronts and the position of the thermal-balance region.

In Setup 2, Beam Sensor 1 responds first because it is nearest the single heater. Sensors farther along the beam respond later and generally reach lower temperatures as heat propagates through the finite-conductance PCB structure while energy is lost to the surrounding air and support structure.

Together, the two configurations support measurements of transient heating response, spatial temperature gradients, thermal time constants, directional heat propagation, dual-source interaction, and the effect of heater PWM power on the beam temperature profile.